## New Driver Test on Daily Vehicle Check

Candidate’s name: ………………………………………	Date:  ………………….

|  | Item | Check | Review | Pass |
| --- | --- | --- | --- | --- |
|  | - Fuel | - Fuel tank full or sufficient for the planned journey. Jerry cans for fuel if necessary. - Fuel sedimentor and filter clear. - No discernible fuel leaks. |  |  |
|  | - Engine oil | - Oil level at, or just below, fill level with vehicle on level ground |  |  |
|  | - Other fluids | - Coolant between minimum and maximum marks in the expansion tank. - Screen washer fluid topped up. |  |  |
|  | - Battery | - Battery connections and holding bracket(s) are secure |  |  |
|  | - Lights | - All lights and indicators function, including interior lights |  |  |
|  | - Wheels & Tyres | - Wheel nuts. Tyre tread depth, tyre damage, especially sidewalls, uneven wear. Spare tyre(s). |  |  |
|  | - Horn | - Does the horn work? |  |  |
|  | - Tools and equipment | - Jack, wheel spanner, first aid kit, seat belts, small tool kit, essential spares (e.g. fan belt), shovel, tow rope, cargo straps, warning triangles, etc. |  |  |
|  | - Brakes | - Hydraulic brake oil; Brakes work |  |  |
|  | - Doors, windows & mirrors | - Doors can be locked. - Windows are clean and can be opened and closed. Wipers work. |  |  |
|  | - Vehicle documents | - Required vehicle documents are in the vehicle or with the driver |  |  |
|  | Start engine | Oil pressure and battery charging lights come on when ignition switch turned on but go off when engine running |  |  |

Assessor’s name and status: …………………

Organization:  …………………………

## New Driver Test on Changing a Wheel

Candidate’s name: ………………………………………	Date:  ………………….

|  | Item | Check | Review | Pass |
| --- | --- | --- | --- | --- |
| - 1 | - Immobilize Vehicle | - Parking brake on. Everyone vacates the vehicle. Wheel chocks placed in appropriate positions. |  |  |
| - 2 | - Position jack | - Jack safely in level position |  |  |
| - 3 | - Wheel nuts | - Wheel nuts slackened |  |  |
| - 4 | - Raise jack | - Wheel lifted off the ground by raising the jack. Vehicle stability checked whilst jacking. - Raised sufficiently to be able to fit spare wheel with fully inflated tyre. |  |  |
| - 5 | - Remove wheel nuts | - Wheel nuts removed and placed in secure place |  |  |
| - 6 | - Remove wheel | - Wheel carefully removed and placed safely to one side. |  |  |
| - 7 | - Replace wheel | - Spare wheel fitted in position and nuts screwed until wheel held securely in position |  |  |
| - 8 | - Lower jack | - Wheel lowered onto the ground and jack removed. |  |  |
| - 9 | - Tighten nuts | - Wheel nuts tightened |  |  |
| - 10 | Wheel & tools | Flat tyre collected and secured. Tools put safely away. |  |  |

Assessor’s name and status: ……………………………….

Organization: …………………………….…

## New Driver Driving Test

Candidate’s name: ………………………………………	Date:  ………………….

|  | Item | Check | Review | Pass |
| --- | --- | --- | --- | --- |
| - 1 | - Starting Engine | - Did driver warm engine before engaging gears and commencing driving |  |  |
| - 2 | - Use of Gears | - Did driver use gears correctly |  |  |
| - 3 | - Reversing | - Did driver reverse in a controlled and accurate manner. |  |  |
| - 4 | - Use of clutch | - Did driver use clutch correctly. |  |  |
| - 5 | - Use of mirrors | - Did driver make use of mirrors |  |  |
| - 6 | - Indicators | - Did driver use indicators when required. |  |  |
| - 7 | - Speed Limits | - Did driver observe speed limits |  |  |
| - 8 | - Traffic signs and traffic lights | - Did driver observe traffic signs and traffic lights. |  |  |
| - 9 | - Braking | - Did driver brake smoothly and use brake effectively. |  |  |
| - 10 | Emergency Stop | Did the driver complete a controlled emergency stop effectively. |  |  |
| - 11 | Handbrake | Did the driver use the handbrake effectively when starting from a stationary position on a slope. |  |  |
| - 12 | Pedestrians | Did driver observe pedestrians and give them sufficient room when passing or crossing the road |  |  |
| - 13 | Other vehicles | Did driver observe other vehicles manoeuvers and give sufficient room when overtaking and when following behind. |  |  |
| - 14 | Driving | Did the driver drive with 2 hands on the wheel. |  |  |
| - 15 | Driving | Did the driver have full control of the vehicle at all times |  |  |
| - 16 | Seat Belts | Did the driver wear a seatbelt and request other passengers to wear a seatbelt |  |  |
| - 17 | Animals | Did the driver observe animals by the side of the road and vary the speed of the vehicle accordingly |  |  |
| - 18 | Driving | Did you feel confident with the driver |  |  |
| - 19 | Parking | Was the driver able to park the vehicle in a safe manner |  |  |
| - 20 | Road conditions | Did the driver drive at a speed suitable for the road conditions. |  |  |

Assessor’s name and status: ……………………………….

Organization: …………………………….…